How to use NCR imaging suite software with EloPOS

Notes:

- Network rules should allow UDP port 137 for NetBIOS traffic

- Network adapter should allow NetBIOS over TCP/IP

- Network shared folder (location to store and deploy captured images)

- Windows server or workstation

- No other DHCP service should exist on the network segment

- Works with and without a network gateway

- When adding additional drivers to Boot.wim, maintained file under 400MB

Imaging Suite Download:
https://www5.ncr.com//support/support_drivers_patches_radiant.asp?Class=Hospitality/GenDrivers_display v5.4.0.4

Content of File:

- Prerequisites

- PXEBOOT-WINPE

- Recovery_Update

- Utilities

- NCR Imaging Suite User Guide.doc

- ReleaseNotes.txt

Instructions:

- Connect server “PXE server” and EloPOS “PXE client” on the same network segment

- On the PXE server, assign an IPv4 static address of 10.10.10.2, subnet mask 255.255.255.0, gateway 10.10.10.1, DNS 8.8.8.8, 8.8.4.4

- Create a network shared folder labeled “Images”

- Go into the folder \PXEBOOT-WINPE\UEFI\ and launch tftpd32.exe

- Configure the TFTP tab with the settings shown below

- Next, configure DHCP tab with the settings shown below

- On the Tftpd32 main window, select the server interface 10.10.10.2

- Go to the folder \ImagingSuite_5.4.0.4\Utilities\ and launch ImagingServer.exe

- Select Capture or Apply Image. In this example, we have a captured image ready.

- Click Choose Share and select the “Images” network shared folder

- Enter credentials allowed to access the shared folder

- Click Browse to select a previously captured image (.wim)

- Click Start

- On the PXE client, enabled PXE boot and UEFI mode in the BIOS settings

- Perform an IPv4 network boot

- Wait for the PXE client to receive a DHCP address from the PXE server

- Wait for PXE client to download boot.wim boot file.

- Wait for PXE client to enter pre-boot environment and load network driver.

- Wait for PXE client to automatically launch ImageClient.exe

- Choose to Connect to NCR server

- If the IP address of the PXE server appears, the PXE client is ready to receive an image.

Other notes:

- When boot.wim is larger than 400MB, EloPOS will crash. Remove unnecessary drivers to reduce the size of the file. Use DISM utility in Windows to modify and cleanup .wim files.

- ImagingServer.exe will use NetBIOS service to communicate with ImageClient.exe